Product Data Sheet

MSBP-2000 (Antiscalent & Sludge Conditioner For Boiler)

MSBP-2000 PLUS offers one pack, multi-functional liquid formulation, for the internal treatment of low medium & high pressure boilers. This product shows effective mineral scale control and sludge conditioning properties

MSBP-2000 PLUS contains :

*	An inorganic phosphate for precipitation of scale forming compounds.

*	A polymer to fluidize and sludge conditioning of the precipitation.

# SPECIFICATIONS:

Appearance				:		Slightly turbid

PH 					:		09.0 – 11.0

Solubility in water 			:		Completely miscible

## Specific Gravity			:		1.12 – 1.14  at 30 C

APPLICATION AND DOSAGE:

MSBP-2000 PLU should be fed in feed water tank. A recommended residual phosphate level must be maintained in blow-down water.

PRECAUTIONS:

# MSBP-2000 PLUS should not be used in water intended for human consumption.

# MSBP-2000 PLUS must be handle with proper safety precautions. For open handling, person should be equipped with rubber gloves, aprons and goggles. Harmful if swallowed. May cause irritation to skin and eyes. Avoid contact with skin, eyes and clothing. Wash thoroughly after handling. Wash empty drums before dispose off.

# FIRST AID:

In case of contact, immediately flush with plenty of water for at least 5 minutes. If swallowed, promptly drink large quantity of water.

PACKAGING:

MSBP-2000 PLUS is available in 35 and 200 Kg packing.

Any suggestion or information contained in this leaflet is based on the evaluation & test methods, which we believe are reliable. These are offered in good faith, but without guarantee, as conditions & method of usage are beyond our control.

It is recommended therefore, that customer must determine the suitability of the product before its use on larger scale.