Product Data Sheet

MSPI-202 ( pH Booster For Boiler)

MSPI-202  is one pack liquid formulation for the boost-up of Hydrate Alkalinity and to maintain desired level of pH level in the system. The need of this product requires due to:

1)	Build-up of Alkalinity or pH in the desired level.

2)	Phosphate treatment program requires certain pH condition in order to initiate the reaction mechanism in the system. Whereas, surplus Hydrate Alkalinity is required to convert magnesium hardness into non sticky sludge.

This product successfully maintains all such operating condition in the limits.

# SPECIFICATIONS

Appearance			:	Slightly turbid

PH					:	11.5 –13.5

Solubility in Water 			:	Completely miscible

Specific Gravity			:	1.2 to 1.4 at 20o C

APPLICATION AND DOSAGE:

MSPI-202 should be fed in feed water tank. A recommended residual phosphate level must be maintained in blow-down water.

PRECATIONS:

MSPI-202  should not be used in water intended for human consumption.

MSPI-202  must be handle with proper safety precautions. For open handling, person should be equipped with rubber gloves, aprons and goggles. Harmful if swallowed. May cause irritation to skin and eyes. Avoid contact with skin, eyes and clothing. Wash thoroughly after handling. Wash empty drums before dispose off.

# FARST AID:

In case of contact, immediately flush with plenty of water for atleast 5 minutes. If swallowed, promptly drink large quantity of water.

PACKAGING :

MSPI-202  is available in 35, and 200 Kgs packing.